AZFP Data Processing Methods

Raw Data

- Copy raw AZFP files and AZFP .cfg file into glider deployment folder

- Format files so each day of deployment is a separate folder (AZFP_data_by_day.py)

- Download delayed mode dataset from ERDDAP and create files of GPS/pitch/roll/depth for glider deployment for use by Echoview (Echoview_File_Formatting_From_ERDDAP.m)

- Create .ecs file for glider deployment for use by Echoview (make copy of the one corresponding to the frequencies on the AZFP, rename after glider deployment ID)

- Download HFR data for deployment timespan from ERDDAP

Echoview/Python

- Create Echoview file that imports the AZFP and glider data, removes background/impulse noise, and removes near-field/below seafloor/glider upcast data (EV_File_Using_Template.py)

- Repick/Recalculate seafloor line for each file (done manually using the lowest frequency Sv data; attempts to code this into the template have all failed for some reason)

- Export cleaned and integrated data for each frequency, including empty cells, in 0.1 nau mi long/1 m deep cells (EV_File_Export.py)

Matlab

- Assign likely species to echoes using dB differencing methods and calculate abundance/biomass based on those species assignments (Echoview_Processing.m)

- Calculate hourly integrated FTLE product over glider timespan and covert to .nc file (fillgaps_MAB_daily_local_all_seasons.m -> backward_ftle_MAB_season_baffin_changeDeriv.m -> ConvertFTLE2nc.m)

R

- Reformat and align AZFP and glider data, and calculate depth-integrated abundances/biomass values over longer horizontal bins (Rutgers_Echosounder_Biomass_Processing.R)

- Make figures using the reformatted/aligned data (Rutgers_Echosounder_Biomass_Plots.R)

- Assign FTLE value to each zooplankton value based on time/lat/long (Zooplankton_FTLE_Processing.R)

- Statistical analysis (Zooplankton_Abundance_Modeling.R)

A combination of Echoview, Python, Matlab, and R was used to process the raw AZFP data from the glider and estimate concentration and biomass numbers of large and small copepods. Data were first offloaded from the glider into a deployment-specific folder, which also included configuration and calibration files, and formatted such that each day of the deployment was its own subfolder. Separately, data from the glider itself were downloaded from the ERDDAP file server and four separate files were created in order to properly format the necessary data for Echoview.

The next processing step was to create Echoview files for each day of the deployment. This was accomplished using Python code that automatically created a new Echoview file from a template, loaded the AZFP and glider data, and saved it to a new file, repeating the process for each day. Once this was finished, the seafloor was manually assigned in each file using Echoview’s best bottom candidate algorithm. Each Echoview file removes background and impulse noise, data from glider upcasts, false data from below the seafloor, and near-field scattering from each frequency (configuration of all algorithms available upon request). A separate Python script was then used to export the cleaned Sv data in cells 0.1 nautical miles in length (along the glider track) and 1 m in depth into one .csv file per frequency per day (either three or four .csv files per day, depending on the AZFP configuration).

Matlab was used to determine the likely species that resulted in the echo from each cell utilizing dB differencing methods. This technique involves examining the difference between corresponding Sv values received at different frequencies within the same integration cell, and assigning that cell a likely species based on whether the Sv difference falls within a range calculated using target strength models of that species. If the Sv difference did not correspond to any species, the “empty” cell was preserved in the dataset. Once species assignments were complete, concentration and biomass were obtained using backscattering cross-section estimates and dry weight values, respectively, and the data were combined over all frequencies and exported into a single .csv file for each day that expanded upon the files exported by Echoview by adding species, concentration, and biomass values for each integration cells.

Finally, data over the entire deployment were imported into R for final clean-up and processing. This process also involved importing the glider data downloaded from ERDDAP as well as bathymetry data from NOAA and marine mammal detection data from robots4whales. R code was used to change the time zone of the data to eastern, time/space align the AZFP and glider data, and prepare the data for various figures.